Technical specifications for plasma cutting machines for Vakarų Metalgama

L5000

Klaipeda, Lithuania 2024

Please provide proposals for new plasma cutting machine, based on parameters listed below.

- Machine (with all its components) must be installed on the track width (from center of old rails) – 5000 mm.

- Machine and all of its components should be built for industrial operations on workload 24/7. Machine should provide maximum stability and undisputable work with near maximum load (taken in to account the manufacturers recommendations on regular maintenance).

- Machine track length - 28000 mm. New track shall be provided by manufacturer. Track rack will be left from old installation. Height – 330mm.

- New cutting table length should be no less 25000mm. Should be able to put 12000mm and 12000mm length plates next to each other. It should be located at the right.

- New cutting table width should be no less then 3400mm. Effective working width no less than 3100 mm, for all operations.

- Cutting table shall be NON vibrating type. The ventilation ducts should open/close with CNC controllers (not pneumatic).

- New plasma source should be installed in area marked “Plasma 2”. This should be its designated please and cannot be moved.

- New ventilation/extraction unit must be proposed. New ventilation/filter place cannot be moved. It must be installed in the place marked “Vent 2”. Connection to filtration system is marked blue – it is underground installed piping. It will be outdoors without any protection from the weather. Extraction volume must be no less than – 9800 m³/h. Filtration efficiency: 99.999% at particles > 5 Micron.

- Plasma marking head should be installed. It shall be connected to plasma bevel unit, moving using plasma head motor. Marking gas – N2 or Argon or Air.

- The machine should be equipped with bevel plasma head. It should have vertical stroke of no less than 350mm. Should be able to produce bevels of no less than +-50°. Degree of C-axis rotation– infinite.

  - Cutting thickness – Mild steel – 50mm (100mm edge start)

  - Stainless steel – 38mm (120mm edge start)

  - Aluminum – 38mm (80mm edge start)

- For plasma cutting, an automated gas mixing console shall be installed. Used gases shall include – O2, N2, air, F5, H2 or Argon – or analog.

- TWO Separate oxyfuel cutting heads should be installed. Used gases shall include – natural gas.  Maximum cutting height for mild steel – 300mm. A height sensor shall be included. Electrical ignition system. Manual bevel angle change by the operator.

- For plate positioning – laser diode will be attached on plasma carriage.

- Inkjet marking system shall be included. This installation will be done on old portal, with track width 5600mm, from old thermal cutting system. Track rack and track itself will be from old machine (15 meter long). Shall include – rotation to part position, height sensing, no less than 16 dot system, new generation controller for writing different fonts and dimensions of letters. It should include an option for marking white color symbols.

- Quotation must include programing software and include programing of all processes – plasma straight, plasma bevel, gas one torch, gas two torches, plasma bevel, inkjet marking and other. Or should be proposed post processor for all processes for existing software Omniwin/Omnibevel.

UAB “Vakarų  Metalgama“

Manager

Tautvydas Ratkevičius

| NEW plasma L5000 electricity consumtion based on average annual production (fill in yellow column) |  |  |  |
| --- | --- | --- | --- |
| Thickness of material, mm | Length of cut, mm | Bevel length, mm | Electricity consumption, kWh |
| 3 | 2653,54 |  |  |
| 4 | 16156,48 |  |  |
| 5 | 7424,44 | 1856,11 |  |
| 5,5 | 59,416 | 14,854 |  |
| 6 | 7479,8 | 1869,95 |  |
| 6,5 | 150,976 | 37,744 |  |
| 7 | 2005,352 | 501,338 |  |
| 7,5 | 719,488 | 179,872 |  |
| 8 | 13876,424 | 3469,106 |  |
| 8,5 | 1599,376 | 399,844 |  |
| 9 | 3419,152 | 854,788 |  |
| 9,5 | 110,544 | 27,636 |  |
| 10 | 21768,048 | 5442,012 |  |
| 10,5 | 10,616 | 2,654 |  |
| 11 | 802,232 | 200,558 |  |
| 12 | 9568,688 | 2392,172 |  |
| 12,5 | 271,512 | 67,878 |  |
| 13 | 959,576 | 239,894 |  |
| 13,5 | 54,432 | 13,608 |  |
| 14 | 4546,336 | 1136,584 |  |
| 15 | 3627,128 | 906,782 |  |
| 15,5 | 134,56 | 33,64 |  |
| 16 | 1393,176 | 348,294 |  |
| 17 | 838,008 | 209,502 |  |
| 18 | 413,976 | 103,494 |  |
| 18,5 | 33,84 | 8,46 |  |
| 20 | 2339,888 | 584,972 |  |
| 22 | 69,68 | 17,42 |  |
| 24 | 14,608 | 3,652 |  |
| 25 | 797,144 | 199,286 |  |
| 30 | 536,88 | 134,22 |  |
| 34 | 109,26 |  |  |
| 35 | 1228,94 |  |  |
| 38 | 10,88 |  |  |
| 40 | 4896,52 |  |  |
| 45 | 225,9 |  |  |
| 50 | 3391,7 |  |  |
| 55 | 294,95 |  |  |
| 60 | 1079,38 |  |  |
| 65 | 6,22 |  |  |
| 70 | 478,95 |  |  |
| 75 | 3,93 |  |  |
| 80 | 77,54 |  |  |
| 90 | 42,94 |  |  |
| 100 | 24,61 |  |  |
| 110 | 2 |  |  |
| 120 | 54,99 |  |  |
| 130 | 76,99 |  |  |
| 150 | 15,84 |  |  |
| 180 | 12,82 |  |  |
| Grand Total | 115869,676 | 21256,324 | 0 |