Technical specifications for plasma cutting machines for Vakaru Metalgama

L5600

Klaipeda, Lithuania 2024

Please provide proposal for new plasma cutting machine, based on parameters listed below.

- Machine (with all its components) must be installed on the track width (from center of old rails) – 5600 mm

- Machine and all of its components should be built for industrial operations on workload 24/7. Machine should provide maximum stability and undisputable work with near maximum load (taken in to account the manufacturers recommendations on regular maintenance).

- Machine track length - 39170 mm. New track shall be provided by manufacturer. Track rack will be left from old installation Height – 340mm.

- New cutting table length should be – 27000mm. Should be able to put two 12000mm length plates next to each other. It should be located at the right.

- New cutting table width should be no less then 3400mm. Effective working width no less than 3100 mm, for all operations.

- Cutting table shall be NON vibrating type.

- New plasma source should be installed in area marked “Plasma 1”. This should be its designated place and cannot be moved.

- New ventilation/filter place can be moved according to the most effective place. It is marked “Vent 1”. It will be outdoors without any protection from the weather. Extraction volume must be no less than – 9800 m³/h. Filtration efficiency: 99.999% at particles > 5 Micron.

- Plasma marking head should be installed. It shall be connected to plasma bevel unit, moving using plasma head motor. Marking gas – N2 or Argon or Air.

- The machine should be equipped with bevel plasma head. It should have vertical stroke of no less than 350mm. Should be able to produce bevels of no less than +-50°. Degree of C-axis rotation– infinite.

  - Cutting thickness – Mild steel – 50mm (100mm edge start)

  - Stainless steel – 38mm (120mm edge start)

  - Aluminum – 38mm (80mm edge start)

- For plasma cutting, an automated gas mixing console shall be installed. Used gases shall include – O2, N2, air, F5, H2, Argon or analog.

- Separate oxyfuel cutting head should be installed. Used gases shall include – natural gas.  Maximum cutting height for mild steel – 300mm. A height sensor shall be included. Electrical ignition system. Manual bevel angle change by the operator.

- For plate positioning – laser diode will be attached on plasma carriage.

- Quotation must include programing software and include programing of all processes – plasma straight, plasma bevel, gas one torch, gas two torches, plasma bevel, inkjet marking, drill and other. Or should be proposed post processor for all processes for existing software Omniwin/Omnibevel.

UAB “Vakarų  Metalgama“

Manager

Tautvydas Ratkevičius

| NEW plasma L5600 electricity consumtion based on average annual production (fill in yellow column) |  |  |  |
| --- | --- | --- | --- |
| Thickness of material, mm | Length of cut, mm | Bevel length, mm | Electricity consumption, kWh |
| 4 | 294,1344 | 0 |  |
| 5 | 895,5954 | 0 |  |
| 5,5 | 57,7836 | 20,553 |  |
| 6 | 25656,2712 | 2013,778 |  |
| 6,5 | 940,527 | 106,47 |  |
| 7 | 9925,9146 | 1106,274 |  |
| 7,5 | 4716,747 | 476,294 |  |
| 8 | 25111,3086 | 3877,64 |  |
| 8,5 | 3458,9016 | 293,488 |  |
| 9 | 8519,8806 | 1275,703 |  |
| 9,5 | 762,7788 | 21,203 |  |
| 10 | 26525,4444 | 1416,792 |  |
| 10,5 | 107,3898 | 24,479 |  |
| 11 | 3593,5326 | 314,938 |  |
| 12 | 18024,5016 | 1532,206 |  |
| 12,5 | 1058,0094 | 119,158 |  |
| 13 | 1352,3328 | 14,872 |  |
| 14 | 3405,8934 | 989,443 |  |
| 15 | 8453,1888 | 903,929 |  |
| 15,5 | 391,2426 | 18,2 |  |
| 16 | 888,4512 | 574,691 |  |
| 16,5 | 169,9236 | 169,234 |  |
| 17 | 2404,4076 | 356,122 |  |
| 18 | 1016,3538 | 45,838 |  |
| 18,5 | 654,4062 | 153,231 |  |
| 19 | 177,3954 | 81,211 |  |
| 20 | 4317,5538 | 757,055 |  |
| 22 | 3,9312 | 0 |  |
| 24 | 333,333 | 165,828 |  |
| 25 | 319,4012 | 0 |  |
| 30 | 146,0088 | 0 |  |
| 35 | 1,1214 | 0 |  |
| 40 | 279,3798 | 0 |  |
| 60 | 3,78 | 0 |  |
| 70 | 87,5448 | 0 |  |
| Grand Total | 154054,37 | 16828,63 | 0 |